The NINTENDO PC INTERFACE SYSTEM is a hardware device designed

several years ago by my brother and myself.  My original

intention was to create a way to transfer game software contained

in Nintendo cartridge ROMs to PC disk files where they could

later be  downloaded to a "universal cartridge", a regular

Nintendo cartridge whose ROMs are replaced with battery-backed

static RAMs.  This relatively simple idea seemed to me the best

solution to the many problems associated with my previous method

which utilized an EPROM programmer and a socketed cartridge.

While this system in its present form works flawlessly on those

first generation Nintendo cartridges for which it was designed,

the newer, larger memory ROM cartridges' diverse addressing

formats made designing a single integrated system a nightmare to

someone of my limited design experience.  Part of the incentive

for going public with this system is to possibly inspire others

to improve upon what we've started.

WARNING!

This is NOT a good electronics project for beginners for several

reasons.  One being that, in these directions, I have assumed the

builder is familiar with the reading of schematics.  These

instructions are far from being detailed.  Secondly, with so many

soldered bus connections, the most experienced assembler is bound

to run into problems which can only be troubleshot with a VOM and

an abundance of patience.  I can just see hundreds of pissed-off

hobbyists writing me nasty letters because of their own crossed

wire.  I tried my best to double-check the accuracy of my

schematics with that of my prototype, but knowing that anything

can happen, I'll be prepared to answer any reasonable requests

for help.

********************

There are five files included in this archived file.

NINTEN.DOC -  This documentation file with instructions and

information (such as it is).

NIN.EXE    -  This is the device driver for the unit written

in assembly language.

SCHEM1.PIC -  The PIC file that displays the schematic for the

PROGRAMMING MODULE.  All the following picture

files can be shown with CSHOW or other graphics

display programs.

SCHEM2.PIC -  The schematic for the UNIVERSAL CARTRIDGE.  This is

a regular Nintendo cartridge that is converted in

the process described below.

SCHEM3.PIC -  The schematic for the INTERFACE BOARD which plugs

into an 8 bit expansion slot and connects to the

PROGRAMMER MODULE via a 25 pin "D" connector.

********************

CONSTRUCTING THE PROGRAMMER MODULE

This is the unit that will actually have cartridges inserted into

it.  I used a plastic 2x3x6 Radio Shack project box.  From the

top edge of one of the 6" sides cut a 1 & 1/8 inch by 4.5" slot.

Now for the bad news.  The 72-pin connector that Nintendo uses is

non-standard.  I had to cannibalize a whole Nintendo System to

use the actual zero insertion force connector built inside the

unit.  This I mounted to the bottom of the project box so that a

cartridge could easily be inserted coming in at a downward angle

then being pressed down to a horizontal position.  Since this

will be minus the spring loaded guiding mechanism of the Nintendo

System, it's important that the 4.5" slot width holds the

cartridge firmly in place while in the "down" position.

The components are mounted on a 2x4 inch perf board.  The D

connector is mounted on the right side of the project box

(slotted being front).  Admittedly, things get a bit crowded.  I

used ribbon cable for bus lines directly soldered to a 72

cardedge (just the "fingers") sawed off a cartridge board.  This

plugs into the lower slot of the connector.  Keep the lines long

enough to wrap around the bottoms and sides of the box to the

component board that will ride on top of the connector.

********************

CONSTRUCTING THE RAM CARTRIDGE

The following is the process used to prep an existing Nintendo

cartridge.  As far as I know any of the games mentioned at the

end of this doc file will work as a "ram cartridge" but to be

sure you may want to use the game cart I did; SOCCER.  The

following directions are listed briefly on the schematic itself.

1) Remove solder bridge (V or H) to CE 22.

2) Remove both ROMs and replace with static rams 43256 (NEC) with

the following pins pulled up: CHR ram - 20,27.  PRG ram -

1,20,27.

3) Scribe a break in the 5 volt circuit path.  Right above pin 16

on the CIC chip is a good place.  Jumper with the 1N4005.  In

the cartridge schematic that diode is the only component

(besides the rams) mounted on the factory board.  All

others, including the battery, I put on a 1.75x1.75" card.

4) Jumper WE pins on CHR ram pin #27 to cardedge #19 and PRG ram

#27 to cardedge #15.  Jumper in the same way pin 1 on the PRG

ram to the punched hole normally reserved for its pin 27.

********************

CONSTRUCTING THE INTERFACE BOARD

The interface board I used was not of my design.  I used the card

plugged into my PC's expansion slot that connects to my MIDI

music system.  This is simply a port address assigning driver

board which means this system uses the same port addresses as a

Roland MPU401 MIDI device.  It should be easy to duplicate using

a breadboard 8 bit PC card and the schematic provided.

********************

RUNNING THE SOFTWARE

Start the program by typing "NIN" at the prompt.  This is the

executable file containing the programmer's device driver written

in assembly language by my brother, Jeff.  For a printed copy of

this code see "FOR FURTHER INFO" at the end of this doc file.

NIN.EXE is a self-explanatory menu-driven program.  To save a

cartridge's software to a disk file:  Place the cartridge to be

copied into the programmer unit.  Press "R" for "Read from port".

This command places the software read from the cartridge's roms

to a buffer in memory.  NOTE: the program responds only to upper

case.  Next press "O".  This command will be followed by a prompt

asking you to name the file that will be created containing the

data in the buffer.

To download software from the disk file to the "ram cartridge":

Firmly install the cartridge into the programmer unit.  Press

"I".  This places data from the file into computer memory.  Next

press "W" to write data to the static rams in the cartridge.

Carefully disconnect cartridge from the programmer.  Set

cartridge DIP switch to horizontal or vertical position depending

upon game.  Cartridge is now ready to be played in your Nintendo

System.

********************

DISCLAIMER

This program and instructions are distributed as is without

warranty.  In no event will I be liable to you for any damages

arising out of the use or inability to use said program or

devices described herein.

********************

MORE INFO

Please keep in mind that, as previously mentioned, this device is

limited to copying only older, smaller memory cartridges.  In

order to compete in the current video game market, Nintendo and

its licensees presently use much larger memory ROMs in their

games which involve dozens of varied addressing schemes.  To

design a single integrated programmer which would accommodate all

these formats would require a much more sophisticated device.

The following, while being far from complete, is a partial

listing of those games that have worked successfully on my

prototype: BASEBALL, GOLF, POPEYE, ICECLIMBER, DUCKHUNT, EXCITE

BIKE, BURGERTIME, PINBALL, HOGAN'S ALLEY, DONKEY KONG JR., URBAN

CHAMPION.

For those of you who successfully complete the building of this

system and would like to become a registered user, I will be

happy to send you a disk containing the latest extended device

driver which allows the programmer unit to accommodate 64K

software cartridges without any change in its hardware.  Just

send a check or money order in the amount of $35.00 to:

Gary Kibler

7902 Holt Avenue #7

Huntington Beach, CA 92647

The following is a partial list of those 64K games that have

successfully loaded on my prototype with the extended driver:

SUPER MARIO, KUNG FU, SLALOM, WRECKING CREW, ICE HOCKEY, SOCCER,

TAGTEAM WRESTLING.

As mentioned before, I am willing to respond to any reasonable

requests for help or information provided you are a registered

user.  Being registered also entitles you to the documentation

and code to both versions of the driver software.  Just mention

this request in your letter.