Windows Platform Design Notes

Design Information for the Microsoft® Windows® Family of Operating Systems

Audio Transform Device Drivers

Abstract

This paper provides information about audio transform device (ATD) drivers for the Microsoft® Windows® family of operating systems. It presents guidelines for audio hardware vendors and device manufacturers to develop drivers for their audio transform devices, which are special-purpose processors that dedicate their computing power to mixing audio streams and adding effects to those streams. Typical effects include encoding and decoding, format conversion, reverberation, echo, chorus, parametric equalization, and three-dimensional (3D) positioning. An audio transform device can boost system performance by offloading the audio processing from the host CPU. Although the operating system provides a baseline set of software-implemented effects, audio transform devices can provide hardware-implemented effects that either are of higher-quality than the software effects or are not available at all as software effects.

Contents

Introduction	3

The Transition to Componentized Audio Drivers	4

System-Level Audio Processing	7

User-Mode and Kernel-Mode ATD Components	11

LEAP Clients	11

LEAP Audio-Processing Graphs	12

Timing Issues for Componentized Audio Drivers	18

Design Guidelines for Hardware and Software	20

Call to Action and Resources	22

This is a preliminary document and may be changed substantially prior to final commercial release of the software described herein.

The information contained in this document represents the current view of Microsoft Corporation on the issues discussed as of the date of publication.  Because Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Microsoft, and Microsoft cannot guarantee the accuracy of any information presented after the date of publication.

This White Paper is for informational purposes only.  MICROSOFT MAKES NO WARRANTIES, EXPRESS, IMPLIED OR STATUTORY, AS TO THE INFORMATION IN THIS DOCUMENT.

Complying with all applicable copyright laws is the responsibility of the user.  Without limiting the rights under copyright, no part of this document may be reproduced, stored in or introduced into a retrieval system, or transmitted in any form or by any means (electronic, mechanical, photocopying, recording, or otherwise), or for any purpose, without the express written permission of Microsoft Corporation.

Microsoft may have patents, patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document.  Except as expressly provided in any written license agreement from Microsoft, the furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property.

Unless otherwise noted, the example companies, organizations, products, domain names, e-mail addresses, logos, people, places and events depicted herein are fictitious, and no association with any real company, organization, product, domain name, email address, logo, person, place or event is intended or should be inferred

© 2003 Microsoft Corporation.  All rights reserved.

Microsoft, DirectMusic, DirectSound, DirectX, Windows, and Windows Server are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

The names of actual companies and products mentioned herein may be the trademarks of their respective owners.

# Introduction

This paper provides information about audio transform device (ATD) drivers for the Microsoft® Windows® family of operating systems. In the next version of the Microsoft Windows operating system, codenamed Windows “Longhorn,” an ATD driver controls an audio transform device that performs hardware-accelerated processing of streams of audio data. By mixing audio streams and adding effects to those streams, the audio transform device can offload audio processing from the host CPU and improve system performance.

Also in Windows “Longhorn,” a new audio API called “LEAP” (short for low-level extensible audio processor) provides application programs with access to the hardware capabilities of audio transform devices. LEAP sits above an ATD driver in the driver stack and converts API calls from a LEAP client into commands to the ATD driver.

Through LEAP API calls, an application program sets up an audio processing graph containing a number of effects to apply to the audio streams that flow through the graph. Typical effects include decoding and encoding, format conversion, mixing, reverberation, echo, chorus, parametric equalization, and 3D positioning. The role of LEAP in graph construction is analogous to that of a plumber who connects sections of pipe together to form paths for water flow.

Depending on the capabilities of the audio transform device, the LEAP module instructs the ATD driver to construct all or a portion of the graph in the audio transform device. Stream processing in this portion of the graph is hardware-accelerated. The LEAP API might need to implement other parts of the graph in software, and these parts consume system resources such as host-CPU bandwidth and system memory. If the audio transform device provides hardware acceleration for all the effects that the LEAP client requires, and the device can also support a sufficient number of audio input and output connections, the entire graph can be mapped to the audio transform device.

In the absence of an audio transform device with an ATD driver, the LEAP client can use a pure software implementation to perform all graph processing. However, this paper focuses on the hardware-accelerated case in which the system contains an audio transform device with an installed ATD driver set.

After constructing the graph, the application feeds raw audio data into the graph for processing and gets the resulting data back for further processing. Some or all of the audio streams in the graph flow into the audio transform device, which processes the streams, mixes them, and routes the result streams back to system memory. After the result streams return to system memory, the LEAP client can process them further. The LEAP client can also choose to play a result stream through an audio-rendering device.

The LEAP API provides two primary services to its clients:

- LEAP allows clients to construct audio processing graphs by assembling effects and buffers.

- After the client completes construction of the graph, LEAP helps manage the flow of audio data through the graph.

Regarding the second point, graph management, LEAP provides the following capabilities:

- Starting and stopping the flow of data through a graph or portion of a graph.

- Changing the data source or destination for a graph input or output.

- Getting and setting control parameters for effects in the graph.

- Monitoring the audio transform device’s read and write cursors, which point into hardware-accessible data buffers located in system memory.

- Clock management for synchronizing the processing performed by one or more hardware devices to a single master clock source.

- Receiving notifications of events and forwarding them to the client.

Typically, an audio transform device is an audio card containing a digital signal processor (DSP) and a direct memory access (DMA) controller. However, other types of hardware designs are also possible. In fact, an ATD driver can control any type of programmable hardware device that performs transformations on audio streams, directly accesses system memory, and follows the design guidelines presented in this paper.

Windows “Longhorn” continues to support the existing hardware-accelerated audio APIs, DirectSound® and DirectMusic®. The DirectSound and DirectMusic components use LEAP internally to perform all audio-transform processing, and they benefit from hardware acceleration if an ATD driver is present. All hardware features that DirectSound can provide in earlier versions of Windows (with the necessary driver support) can be made available on Windows "Longhorn" with a suitable ATD driver. Examples include extended property sets like I3DL2 (IASIG Interactive 3D Audio Rendering Guidelines Level 2) and EAX (Creative Technologies' Environmental Audio Extensions).

For backward compatibility, Windows “Longhorn” can use audio rendering devices and drivers that work in Windows XP, Windows Server™ 2003, and earlier. However, Windows “Longhorn” treats these devices as simple, single-stream renderers without hardware mixing or effects. Thus, older drivers that provide DirectSound or DirectMusic hardware acceleration in earlier versions of Windows are not hardware-accelerated in Windows “Longhorn.” The way to achieve hardware acceleration in Windows “Longhorn” is through an ATD driver.

No plans exist to provide service packs to add ATD driver support to Windows XP, Windows Server 2003, or earlier.

# The Transition to Componentized Audio Drivers

In Windows XP, Windows Server 2003, and earlier, the audio driver model fuses audio-transformation and audio-rendering functions into a single device. In Figure 1, the transform and rendering stages reside in the same audio device.

Figure 1 – Old Audio Driver Model Fuses Transform and Rendering Stages

In this figure, the audio device reads one or more audio streams from system memory into the transform stage. The result stream from the transform stage flows directly into the rendering stage and is not accessible to the host CPU. Consequently, an audio application cannot process an audio stream further after it sends the stream to the hardware. Thus, if an application requires the intermediate results of any audio stream processing, it has no option but to perform the processing in software before the stream enters the audio device. Hardware acceleration is not an option.

Another effect of fusing the transform and rendering stages of an audio device is to make hardware designs more complicated. A hardware vendor whose primary interest is in transforming audio streams has no choice but to perform audio rendering as well.

In Windows “Longhorn,” audio drivers conform to a componentized model that treats the transformation and rendering functions as independent devices. The model splits audio drivers into separate transformation and rendering components to manage these devices. The hardware for the logically separate transform and rendering devices can reside on the same physical audio card or on separate cards.

In Figure 2, the audio-transform hardware and audio-rendering hardware appear as two independent devices.

Figure 2 – New Componentized Audio Driver Model

The audio transform device reads the input streams from system memory, applies audio effects and mixing, and writes the result streams back to system memory, where they are accessible to the host CPU for further processing. At the bottom of Figure 2, the rendering device reads the output stream from system memory and plays it.

Audio transform devices can be simpler because they no longer need to include audio-rendering hardware. In addition, Microsoft simplifies development of audio-rendering devices by providing drivers for devices that conform to the Universal Audio Architecture (UAA) hardware standard. By definition, a device that complies with the Intel “Azalia,” USB Audio, or IEEE 1394 audio specifications also conforms to the UAA standard.

As in Windows XP, Windows Server 2003, and earlier versions of Windows, audio-rendering drivers in Windows “Longhorn” still conform to the kernel streaming (KS) standards, which specify the use of the KS filters, KS pins, and topology nodes that are familiar to Windows audio driver writers. However, ATD drivers do not need to conform to the KS model. (Nothing prevents developers from using aspects of the KS model for their internal implementations if they prefer to do so.) Instead, ATD drivers can use a simpler model that explicitly specifies the construction of audio processing graphs containing effects and buffers.

Some currently available audio cards combine audio-transform and audio-rendering functions on the same audio card. To adapt these cards to the componentized audio model, hardware vendors can develop separate ATD drivers and rendering drivers to represent these functions as logically separate transform and rendering devices. This approach works as long as the audio hardware satisfies the following requirements:

- The transform function can stream results back to system memory.

- The transform function operates independently of the rendering function.

In Windows “Longhorn,” an ATD driver cannot use an audio transform device that fails to meet these two requirements. Regarding the second requirement, drivers can sometimes hide hardware dependencies that might exist between the transform and rendering functions, but the drawback to this approach is that it tends to make the drivers more complicated.

In Figure 2, the audio transform device reads input data from system memory and writes result data back to system memory. Moving audio streams across hardware-software boundaries requires synchronization and buffering, which inevitably add to the latency of the stream. Several techniques can be used to mitigate this latency:

- Windows “Longhorn” supports a new mode of thread scheduling that offers guaranteed scheduling and guaranteed CPU time slices. LEAP can use this mode for scheduling the LEAP thread that moves data through the software portion of an audio processing graph. By reducing uncertainties in thread execution times, buffers can be made much smaller, reducing latencies.

- If an audio transform device is able to directly access and interpret audio data in the application’s input and output buffers, no LEAP thread is needed as act as intermediary between the application and the device, eliminating a potential source of latency.

- A LEAP application can choose to perform all processing of one or more time-critical streams in software, bypassing the audio transform device altogether. All other streams should use hardware-accelerated processing, if available, to avoid unnecessarily burdening the host CPU.

- A LEAP application that requires sub-millisecond latency (for example, a "pro audio" authoring tool) can configure the audio rendering device to operate in "exclusive mode." By default, the global audio engine (described in the next section) feeds a global mix to the rendering device, but in exclusive mode, the application provides audio data directly to the rendering device, and global mixing is disabled. Result data from the audio transform driver must return to system memory before being rendered, but the latency added by this round trip can be eliminated by configuring the rendering driver to read directly from the buffers to which the audio transform device writes the result data. If both the audio transform and rendering device provide the necessary hardware capabilities, and the LEAP graph contains no software effects, exclusive mode can maintain latencies below the one-millisecond threshold.

The first three techniques are discussed later in this paper.

# System-Level Audio Processing

Figure 3 is a system-level diagram showing the division of labor between the audio transform device and the audio rendering device in more detail.

Figure 3 – System-Level Audio Driver Architecture

Located above the LEAP module is an audio application that configures the audio processing graph by calling into the LEAP API. The LEAP module converts the API calls that specify the hardware portion of the graph into driver commands that it passes to the ATD driver.

To improve performance and system stability, the ATD driver consists of two parts:

- A user-mode part interfacing to the LEAP module

- A kernel-mode part controlling the audio transform device

The job of the user-mode part of the ATD driver is to avoid unnecessary transitions between user and kernel mode by handling some commands from the LEAP module completely in user mode, and by batching the remaining commands before passing them to the kernel-mode part of the ATD driver. The kernel-mode part is responsible for directly controlling the audio transform device.

The LEAP module interfaces to the ATD driver through a Microsoft-specified ATD device driver interface (DDI). The DDI is a COM interface with a set of methods that the LEAP module calls to send commands to the user-mode part of the ATD driver. The user-mode and kernel-mode ATD components communicate through a custom software interface, which hardware vendors can define to fit their individual needs. Note that the LEAP module uses the ATD driver to configure the hardware-resident portion of the audio graph, but after construction of the graph completes, the application’s input and result streams can travel to and from the audio transform device without intervention from LEAP.

In Figure 3, the audio streams entering the audio transform device are labeled “Audio Inputs,” the stream exiting it is labeled “Audio Output.” The figure shows only a single audio output stream, which is typical. However, some applications might construct graphs with multiple audio output streams.

The LEAP application maps some parts of the audio processing graph to hardware and other parts to software:

- LEAP implements the hardware portion of the graph in the audio transform device. At the bottom of Figure 3, the audio input streams enter the audio transform device, traverse the hardware-resident portion of the graph, and emerge from the device as the audio output stream.

- In Figure 3, the audio input and output streams that pass through the LEAP module undergo processing in the software-resident portion of the graph. LEAP uses its library of software-emulated effects to implement the software-resident portion of the graph.

Some round trips through the graph might traverse a combination of hardware effects and software effects. Other round trips might traverse a software-only or hardware-only portion of the graph.

A graph might reside entirely in hardware or in software:

- An all-hardware graph resides entirely in the audio transform device, in which case the audio input and output streams in Figure 3 flow directly between the application and the audio transform device, bypassing the LEAP module altogether.

- The LEAP client implements an all-software graph entirely in software by assembling elements from its library of software effects. In an all-software graph, the audio input and output streams in Figure 3 flow between the application and LEAP module, bypassing the audio transform device.

In Figure 3, LEAP connects the audio transform device’s audio output stream to the global audio engine. The audio engine is the system component that mixes the audio streams from all application programs and feeds the resulting output stream to the audio rendering device to be played.

Figure 3 shows the audio output stream branching to two destinations:

- The stream returns to the LEAP application for further processing.

- The stream flows to the global audio engine to be played through the audio-rendering device.

Not all LEAP applications direct the audio output stream to both destinations. For example, an application might read raw data from an input file, process it, and write it to an output file without playing the data. Another application might play the audio output stream but perform no further processing of the stream.

No direct data path exists between the audio transform device and rendering device. After being processed by an audio transform device, a stream must reenter system memory before it can be played by the global audio engine.

Audio applications that need to play only a single audio stream with no effects can use the Streaming Audio Renderer (SAR) API instead of the LEAP API. In Figure 3, some of the application streams arriving at the global audio engine might come from SAR applications, as well as from audio sources such as CD players. A Streaming Audio Capture (SAC) API is also available for recording data from audio input devices such as microphones, but Figure 3 shows no capture streams.

In Windows “Longhorn,” the global audio engine replaces much of the functionality of the kernel mixer, KMixer.sys. KMixer is the software component that performs global mixing in Windows XP, Windows Server 2003, and earlier. By default, KMixer performs mixing for all rendering streams. For example, if application A generates 3 streams and application B generates 5 streams, KMixer mixes all 8 streams. In contrast, the LEAP module mixes all the streams within an application, and the audio engine typically receives only one result stream from each application. For example, assume that applications A and B again generate 3 and 5 streams, respectively. Because the LEAP module does most of the mixing, the audio engine mixes only the two result streams from these applications. In a system with an audio transform device, mixing by the LEAP module is hardware-accelerated, which usually reduces the processing load on the host CPU.

In Windows "Longhorn," the operating system provides a special thread-scheduling mode that guarantees thread activations within regular intervals and reserves a portion of the host CPU’s bandwidth for each activation. The audio engine takes advantage of this feature to run its service thread at shorter and more uniform intervals than would otherwise be possible. Thus, the audio engine can use smaller buffers and adds significantly less latency to audio streams than KMixer, which does not have access to real-time services. KMixer must compensate for scheduling uncertainties by allocating bigger buffers, which increases latency.

In Windows XP, Windows Server 2003, and earlier, DirectSound applications are able to avoid KMixer’s latency by making use of the hardware-accelerated mixing that some audio devices provide. In addition, an audio device can add hardware-accelerated 3D effects to a stream before mixing it. After a DirectSound hardware-accelerated stream flows into an audio device, however, it passes directly from the audio-transform hardware to the audio-rendering hardware and is no longer accessible to the host CPU.

Because DirectSound hardware-accelerated streams bypass KMixer, the KMixer output stream represents only a portion of the audio that plays through the speakers. The operating system must rely on the hardware vendor’s audio driver to apply global controls to hardware-accelerated streams, and some drivers might implement the controls incorrectly or not at all. In Windows “Longhorn,” all rendering streams flow through the global audio engine. Thus, the audio engine can reliably apply global controls to the output stream.

The system can apply global controls to both the input streams and output streams of the global audio engine. The following are examples:

- Effects such as acoustic echo cancellation (AEC) need access to the global output stream that plays through the speakers in order to cancel the speaker sound from the microphone capture stream.

- The audio engine can apply system audio policies to its input streams. An example is audio ducking: if two of the input streams to the audio engine are from the CD player and telephony headset, the system can automatically turn down the volume on the CD player when a phone call comes in so that the user can hear the conversation clearly.

# User-Mode and Kernel-Mode ATD Components

Because transitions between user and kernel mode require expensive context switches, the user-mode part of the ATD driver tries to minimize kernel-mode transitions as follows:

- Some requests from the LEAP can be handled completely in user mode. For example, the user-mode part of the ATD driver can store static information such as the audio transform device’s friendly name, the vendor’s company name, and the device’s general capabilities. If LEAP asks for this information, no kernel-mode transition is required to handle the request.

- The kernel-mode part of the ATD driver can map one or more key hardware registers into virtual-memory addresses that the user-mode part can access. Thereafter, if LEAP requests information from one of these registers, the user-mode part of the ATD driver can read the register directly without incurring the overhead of a kernel-mode transition.

- The user-mode part of the ATD driver can frequently batch several commands from LEAP into a single message that it sends to the kernel-mode ATD component. The message incurs only a single transition to kernel mode and back. To simplify the batching mechanism in the ATD, LEAP batches its commands together before handing them to the user-mode ATD component.

Regarding the second point, LEAP applications continually monitor an audio transform device's read and write cursors to determine how much data the device has read from or written to the various audio buffers in system memory. A device might use a set of position registers to keep track of the current cursor values. Because the ATD driver handles so many cursor queries, mapping the position registers to user mode can significantly improve performance. When LEAP forwards a cursor query from the application to the ATD driver, the driver's user-mode component can directly read the appropriate position register – without the need for a kernel-mode transition – and translate its value to the form that the query requires.

In contrast with drivers that reside entirely in kernel mode, a significant portion of an ATD driver operates in user mode. Debugging of the user-mode ATD component’s interactions with the LEAP module requires only user-mode debugging, which software developers are frequently more familiar with than kernel-mode debugging.

Developers should consider placing most of the complexity of the driver in the user-mode part. This approach can improve system stability by making the kernel-mode part simpler and less prone to errors that cause the system to stop responding.

# LEAP Clients

In Windows “Longhorn”, the Microsoft audio architects’ philosophy is to provide applications with explicit control over construction of audio processing graphs. Method calls in the LEAP API have no hidden side effects. The LEAP client makes all decisions regarding the order in which each part of the graph applies its effects, the amount of buffering in each stream, and which parts of the graph reside in hardware and in software. The LEAP module does not attempt to override or second-guess these decisions.

Similarly, ATD drivers should implement the specified graphs as faithfully as possible. However, if an audio transform device is unable to internally configure its hardware to conform precisely to the graph specified by the LEAP client, the ATD driver has some leeway to substitute a graph that is, for all practical purposes, functionally equivalent.

In Windows XP, Windows Server 2003, and earlier, control over graph construction is more limited:

- The SysAudio system driver is responsible for building audio graphs, which it constructs automatically from the available hardware and software components and exposes to applications as virtual audio devices.

- KMixer automatically adjusts the amount of output buffering based on whether it determines that data starvation is imminent.

- KMixer performs several automatic operations on its input data that can lead to unexpected or unwanted results in some cases. Applications have no control over KMixer's construction of the mixing graph, its choice of format conversions and sample-rate conversions, or its choice of a format for the final mix.

Applications that need to circumvent this type of “help” from the system are not always able to do so.

On the other hand, some application programs might not require a high degree of control over graph construction. This type of application does not need to interface directly to the LEAP API. Instead, it can interface to a higher-level audio API that takes a simpler description of what the audio application requires and translates it into the detailed commands that the LEAP API needs to configure the sequence of effects for each stream. (Additional information on higher-level audio APIs will be forthcoming in future announcements from Microsoft.)

The LEAP client, which can be either an audio application or a high-level audio API, needs enough intelligence to recognize the effects that an ATD supports and configure them appropriately.

The simplest type of LEAP client relies on prior knowledge of the audio effects that it requires. (Most games fall into this category.)  The client also knows the buffering requirements for each effect:

- How many input and output buffers the effect requires

- The minimum size required for these buffers

- Whether the effect can perform in-place processing

An effect that performs in-place processing reads the data from the input buffer or buffers, modifies it, and writes the data back to the same buffer or buffers. An effect that does not perform in-place processing reads data from one buffer (or set of buffers) and writes the result to a different buffer (or set of buffers). Some effects might support both modes of operation.

An interactive LEAP client can obtain information about the effects available at run time (for example, effect names, input and output buffering requirements, and parameter names, types and ranges) and present this information to the user through a graphical user interface (GUI). In this case, the LEAP client relies on the user to provide the knowledge necessary to construct the graph. An audio authoring tool typically belongs to this category.

# LEAP Audio-Processing Graphs

Through the LEAP API, an application constructs an audio processing graph that is composed of two types of element:

- FXBuffers – Intermediate storage for audio data that is in transit through the graph

- Effects – Operations performed on audio data

FXBuffers can be classified according to whether they reside in system memory or in the audio transform device:

- Software FXBuffers – Reside in system memory.

- Hardware FXBuffers – Reside in the audio transform device’s memory.

Similarly, effects can be classified according to whether they are implemented in software or hardware:

- Software Effects – Implemented (in LEAP or by third parties) as independent COM objects called audio processing objects (APOs), which execute on the host CPU.

- Hardware Effects – Performed by the audio transform device.

LEAP represents every discrete operation that a graph performs on an audio data stream as an effect. Any graph element that reads or writes data is an effect.

A typical graph might include the following effects:

- The transfer of raw audio data from one of the application’s input buffers to an FXBuffer at a graph input

- Format conversions, SRC, and clock-drift correction

- Mixing of audio streams

- The transfer of processed audio data from an FXBuffer at a graph output to one of the application’s output buffers or to the global audio engine

The effects that can appear in an audio processing graph form the following hierarchy:

- Processing Effect – Operates only on audio data contained in an FXBuffer or a set of FXBuffers (for example, chorus or mix). All the FXBuffers that an instance of a processing effect reads from or writes to must be of the same type (software or hardware).

- Connector Effect – Moves audio data to or from the graph or across a software-hardware boundary in the graph.

  - Voice – Moves audio data from a LEAP application’s input buffer to an FXBuffer in the graph, and performs some initial processing of the data.

    - Static Voice – Reads audio data sequentially or cyclically from a single buffer whose contents typically remain unchanged as long as the buffer stays in active use.

    - Streaming Voice – Reads audio data from a queue of relatively small buffers that the application must replenish as the voice successively consumes the buffers.

  - Return – Moves audio data from FXBuffers in the graph to application buffers.

  - Bridge – Moves audio data from a software FXBuffer to a hardware FXBuffer, or vice versa.

  - Output – Moves audio data from a software FX buffer to the global audio engine, which executes in a separate service process.

Figure 4 represents an audio graph that accepts input audio streams from a LEAP application and returns a single result stream to the application.

Figure 4 – Audio Processing Graph with Software and Hardware Effects

This graph combines hardware and software effects. Effect1 and Effect4 are software effects. Effect2 and Effect3 are hardware effects.

A simple effect (for example, Effect1, Effect2, and Effect4 in the figure) has one input stream and one output stream. A more complex effect can have more than one input or output stream. For example, Effect3 is a mixer that accepts several input streams and generates a single output stream. Typically, an effect requires one or more input and output streams, but some effects might lack either inputs or outputs (although the figure contains no such effects). Effects can also be classified according to their buffering requirements. Effects that perform in-place transformations on data use the same set of buffers for input and output. Other effects require separate sets of input and output buffers.

In Figure 4, the LEAP voice and a LEAP return manage data at the beginning and end of the graph:

- The LEAP voice converts the audio data in the application’s input buffers from the application’s stream format to the format that the audio transform device uses internally. In addition to performing sample-rate conversion (SRC), the voice might need to compensate for drift between the application’s master clock and the audio transform device’s internal hardware clock. Drift compensation is unnecessary if the application chooses to use the hardware clock as its master clock.

- The LEAP return converts the result data from the audio transform device back to the application’s stream format and writes the data into the application’s result buffer. The LEAP return performs SRC and might need to compensate for clock drift as well.

In this example, LEAP implements the voice and return elements in software.

However, an audio transform device has the option of implementing format conversion and SRC in hardware to offload this processing from the host CPU. For example, to replace the software voice with a hardware voice, the audio transform device must be able to read data directly from the application’s input buffer. The application should ask the ATD driver to allocate this buffer to ensure that it lies in a portion of the system memory that the audio transform device’s DMA controller can access. By eliminating the need for a LEAP thread to perform the voice processing in software, an audio transform device removes one source of latency from the audio graph. An example of an all-hardware graph appears later in this section.

The LEAP thread that moves data through the software portions of the graph schedules itself to run at regular intervals. The interval length is referred to as a quantum. The amount of audio data that moves through this part of the graph during one quantum depends on the sample rate and data format. For example, consider a 10-millisecond quantum and a stream with a 44.1-kHz sample rate, two channels, and a 32-bit sample size. In this case, the amount of data processed in one quantum is

(10 milliseconds)*(44.1 kHz)*(2*4 bytes) = 3528 bytes

A quantum’s worth of data is a convenient size for specifying buffer capacity in the software portion of an audio processing graph. The size an FXBuffer in system memory is either one or two quanta. An FXBuffer that is attached to the software side of a bridge (or hardware voice or return) always holds two quanta. All other FXBuffers in system memory hold one quantum. If an effect in the software portion of the graph accepts input data in one format and produces output data in another format, the amount of data corresponding to a quantum is different at the effect’s inputs and outputs.

In the hardware portion of a graph, the concept of a quantum is less useful. If an audio transform device consists of a DSP that schedules itself to run at regular intervals, this interval is the hardware quantum. A hardware quantum is typically smaller than the software quantum, which is the scheduling interval for the LEAP thread. To simplify buffer handling at the software-hardware boundary, the length of the software quantum should be an integral multiple of the length of the hardware quantum. However, the hardware design for audio transform device is not constrained to follow this model, and for some designs the concept of a hardware quantum might not be relevant.

In Figure 4, the dashed line through the voice and return elements indicates the division between the client application and LEAP. Everything to the left belongs to the application domain, and everything to the right is part of the LEAP domain. The LEAP domain contains the audio processing graph. LEAP implements one portion of the graph in software, and the other in hardware.

In Figure 4, the input and output bridges straddle the software-hardware boundary to indicate that the data crosses this boundary inside the bridges. A bridge is always a hardware effect. The audio transform device implements the bridge functions and uses its bus-mastering capabilities to transfer data across hardware-software boundaries. To make these transfers more efficient, the data on the software side of each boundary crossing is double-buffered. In other words, the data “ping-pongs” between the two halves of an FXBuffer: while the LEAP thread is moving a quantum of data to or from one of the two halves, the DMA controller is moving data from or to the other half. The ATD driver allocates these buffers from system memory that is accessible to the DMA controller.

To more clearly indicate whether an effect performs in-place processing, Figure 5 uses a different set of graphical conventions to represent the audio graph in Figure 4. In this representation, the only blocks in the diagram are the buffers themselves. The figure represents audio effects as arrows.

Figure 5 – Buffer-Centric Representation of Graph in Figure 4

The looped arrows representing Effect1, Effect2, and Effect4 indicate that these effects perform in-place processing on the data in FXBuffer1, FXBuffer2, and FXBuffer4, respectively. In contrast, Effect3 takes data from several input buffers (including FXBuffer2) and mixes the result into FXBuffer3.

In Figure 5, the slashed lines inside the four FXBuffers indicate that they are double-buffered. FXBuffer1 and FXBuffer4 reside in system memory and hold two quanta of data each. The ATD driver allocates both FXBuffers to ensure that they are accessible to the audio transform device. The application allocates the input buffer and output buffer, which it shares with the LEAP thread. Each time the LEAP thread executes, it moves a quantum of data from the input buffer to FXBuffer1, and it moves another quantum of data from FXBuffer4 to the output buffer.

In Figure 5, Effect1, Effect4, the LEAP voice, and LEAP return are all software effects. If the hardware is capable of implementing these effects, the LEAP thread is unnecessary and the graph latency is thereby reduced. Figure 6 shows an all-hardware version of the same graph.

Figure 6 – An All-Hardware Graph

In the all-hardware graph, the audio transform device implements the LEAP voice and return in hardware. The audio transform device performs the conversions between its internal stream format and the application’s stream format.

The vertical dashed line marking the software-hardware boundary also marks the division between the application’s domain and the LEAP domain. LEAP implements the entire graph in hardware.

Figure 7 shows a buffer-centric representation of the all-hardware graph in Figure 6.

Figure 7 – Buffer-Centric Representation of Graph in Figure 6

In this graph, Effect1, Effect2, and Effect4 are in-place effects, as in the previous example. However, in this version of the graph, all three are hardware effects. Effect3 is again a hardware effect that reads from several input buffers and writes to a separate output buffer. Only two FXBuffers are needed because Effect1 and Effect2 perform in-place processing of the data in FXBuffer1, and Effect4 performs in-place processing of the data in FXBuffer2.

In Figure 7, the arrows labeled “LEAP voice” and “LEAP return” represent hardware transfers between system memory and the audio transform device. The device directly reads the data that the application writes to its input buffer. The device also writes directly to the application’s output buffer. The application allocates the system memory for its input and output buffers through the ATD driver to ensure that the buffers are hardware-accessible. In order to know how much data to read and write, the application thread monitors the device’s read and write cursors. The read cursor points to the next block of data in the input buffer that the device will read. Similarly, the write cursor points to the next block of data in the output buffer that the device will write. The application requests the cursor positions through the LEAP API, and LEAP forwards the queries to the ATD driver.

Figure 8 shows an audio processing graph that applies a hardware effect to a stream before playing the stream through the global audio engine and rendering device.

Figure 8 – Graph Combining Audio Processing and Audio Rendering

In this example, the LEAP application performs no further processing of the stream after it passes through the hardware effect.

The box labeled “Output Connector” is a software effect that transports the output stream from the application’s process to the service process that the global audio engine runs in. By opening more than one output connector, a LEAP application can send additional output streams to be played through the renderer.

# Timing Issues for Componentized Audio Drivers

The componentized audio model depends on the real-time scheduling mode in Windows “Longhorn” to keep the latencies of audio processing graphs small.

Figure 9 indicates the type of scheduling guarantees that Windows makes for a real-time thread.

Figure 9 – Thread Activations with a Quantum of 10 Milliseconds

In this example, the thread’s scheduling quantum is 10 milliseconds. The operating system guarantees that a real-time thread executes once during each quantum. However, this guarantee does not mean that successive thread activations necessarily occur at uniformly spaced intervals of one quantum. Each X in the figure represents a thread activation, which can occur anywhere within a particular quantum. For this example, the first quantum begins at 0 milliseconds, and successive thread activations occur at 1, 15, 29, 34, and 41 milliseconds.

During initialization of the audio system, a LEAP client can request a particular quantum length for the LEAP thread that is responsible for moving data through the software part of the audio processing graph. The operating system grants the request only if it can reserve the necessary host CPU bandwidth for the thread. Selecting the right quantum length requires balancing of an application’s latency requirements against overall system performance. A smaller quantum length reduces graph latency but is more costly in terms of CPU bandwidth.

As explained previously, each time an audio graph crosses a software-hardware boundary, buffering is necessary on both sides of the crossing to handle the synchronization of data between subsystems with different clocks. Each crossing adds one quantum of latency to the graph.

Without real-time scheduling, the lengths of the intervals between activations of the LEAP thread that processes the software portions of the graph are more variable. The worst-case interval length might increase with system loading and be hard to predict from known parameters. Faced with these difficulties, the only practical way to reduce the risk of data starvation is to add more padding to the buffers and verify through testing that the audio system operates glitch-free over some range of system loading. However, increasing the buffer size enough to achieve reliable operation in these conditions can result in large graph latencies.

By improving the predictability of thread scheduling, the real-time services in Windows “Longhorn” help to make the latencies across software-hardware boundaries significantly smaller. The resulting latencies should be small enough to meet the needs of nearly all audio applications. Occasionally, however, an application might have one or more time-critical audio streams that cannot tolerate even these reduced latencies.

For example, to heighten a computer game’s interactive feel, some games require certain “twitch” sounds, such as pulling a trigger, to occur within one or two video frames of a user input. In a system with componentized audio, the way to achieve the absolute minimum latency for these time-critical streams is for them to bypass the audio transform device. In other words, these streams forgo hardware acceleration in order to reduce their latencies. The LEAP client should still submit its other, less time-critical streams to the audio transform device for processing to avoid unnecessarily loading down the host CPU. The resulting audio processing graph contains some data paths that flow through the audio transform device and others that bypass it. A software mixer can combine the result stream from the audio transform device with the time-critical streams so that the application can provide a single output stream to the global audio engine.

Relying on software instead of hardware to process a time-critical stream places a greater processing burden on the host CPU. An application can reduce the load on the host CPU by following these rules:

- Allow only time-critical streams to bypass the audio transform device.

- Do as much pre-processing of the time-critical stream as possible before it enters the audio graph.

Regarding the second point, a typical twitch sound in a game might be stored in a static buffer, converted to the same format as the result stream from the audio transform device, and use only static effects, which are effects that the application applies to the data in the static buffer before playing it.

# Design Guidelines for Hardware and Software

Hardware vendors can improve the performance and capabilities of their audio transform devices by following these guidelines for hardware design and ATD driver development.

First, an audio transform device should include a capable DMA controller to transfer input streams from system memory to the audio transform device’s memory, and to transfer the result stream back to system memory. Even in a 64-bit system, a 32-bit DMA address range is probably sufficient because the ATD driver can ensure that the FXBuffers that it allocates in system memory are accessible to the DMA controller. The FXBuffers might also need to conform to the DMA controller’s memory-alignment requirements.

ATD driver developers can implement as many or as few audio effects as they choose for their audio transform devices. A potential client can determine whether to use an audio transform device after querying the ATD driver for the list of effects that it supports in hardware.

Some audio transform devices might be targeted narrowly toward a particular type of application. For example, the ATD driver for an AC-3 decoder might support only AC-3 decoding and be used only by applications that play AC-3 audio streams.

To be useful to a wider range of applications, an ATD driver should implement at least a minimum set of effects. For example, typical LEAP clients expect ATD drivers to implement the same base set of audio effects that LEAP provides through software emulation in the absence of an audio transform device. To serve as an effective LEAP hardware accelerator, an ATD driver should support the following effects:

- Chorus

- Flanger

- Echo

- Distortion

- Compressor

- Parametric equalization

- I3DL2 reverberation

Computer game applications frequently use some or all of these effects. Microsoft DirectX® 8.0 defines the same set of effects for DirectSound media objects (DMOs). For more information about DMOs, see the Microsoft Platform Software Development Kit (SDK) documentation.

One optimization that a LEAP client can use to eliminate unnecessary SRC and format-conversion processing is to configure the audio transform device to use the same data format that the global audio engine uses for its output stream to the rendering device. If an audio transform device can claim support for several stream formats, the LEAP client has a wider range of potential formats to choose from and is more likely to find a format matching that of the rendering device.

A similar optimization is for the LEAP client to use for its own data the same stream format that the audio transform device uses. This optimization is not as important if the audio transform device provides hardware-accelerated format conversion. If the audio transform device does not provide hardware conversion, this optimization can eliminate most but not all software processing. Some software tweaking might still be necessary to compensate for drift between the LEAP master clock and the audio transform device’s hardware clock. The application can avoid the need for drift compensation by using the hardware clock as its master clock.

Even if an audio transform device does not use a particular stream format internally, it can still claim support for that format as long as it performs hardware-accelerated conversion between that format and its own internal format.

A hardware-accelerated voice effect should have capabilities similar to those of the LEAP software-emulated voice. These capabilities include the following:

- Format conversion from a range of PCM formats (see the following comments) to any of the audio transform device's supported internal formats

- SRC

- Low-pass filtering

- Matrix mixing from one source stream to any number of destination streams (which can all have different numbers of channels)

- Simple linear parameter curves on the transformed audio data so that synthesizer envelopes can be implemented

- A position register to keep track of how much data the DMA controller has read from the buffer

Ideally, the range of supported PCM formats should cover frequencies from 100 Hz to 200 kHz, any number of channels, and samples with int8, int16, int20, int24, int32, and float32 values. At a minimum, the input bridge should handle input streams with the following parameters:

- Samples with int8, int16, and float32 values

- Mono and stereo

- Sample rates of 8 kHz, 11.025 kHz, 22.05 kHz, 44.1 kHz, and 48 kHz

A hardware-accelerated return effect should have similar capabilities.

When designing an in-place effect, a hardware vendor might, without much difficulty, be able to implement a second version of the same effect that uses separate input and output FXBuffers. A LEAP application can use this version of the effect to both transform the input data and move the modified data to the output buffer. Without this capability, the LEAP application can achieve the same result only by inserting a second effect into the graph to transfer the results from the input buffer to the output buffer after the first effect completes its in-place transformation.

The ability to start and stop multiple voices simultaneously is a useful feature for an audio transform device. When the user hits the "play" or "stop" button, all the voices should synchronously begin or end at the same sample time. For example, precisely synchronizing two streams is helpful when mixing two versions of the same sound. To control the degree to which an effect such as chorus is applied to a sound, one technique is to add a "dry" stream containing the original sound to a "wet" version that has been transformed by the effect. Allowing these two streams to fall out of phase with each other by even a few samples can produce unwanted audio artifacts similar to reverb.

A composite audio card might contain an audio transform device in addition to other devices. The hardware design should allow the operating system to control the audio transform device independently of the other devices to ensure that power management and Plug and Play work smoothly.

Each LEAP application program has its own instance of the user-mode part of the ATD driver, but all applications share a single instance of the kernel-mode part of the ATD driver, which controls the audio transform device. In this way, a single audio transform device can be shared among several audio applications.

The kernel-mode part of an ATD driver should place a limit on the number of instances of the ATD device interface that applications can create. The available hardware resources – chiefly, DSP processing bandwidth and DSP memory – determine this limit. Depending on the hardware capabilities of the audio transform device, the driver has the option of exposing the available resources in one of the following two ways:

- As several instances of the same type of logical audio transform device with identical capabilities

- As instances of more than one type of logical audio transform device with differing capabilities

As discussed earlier, the user-mode part of the ATD driver communicates through a custom interface to the kernel-mode part of the driver. To avoid compromising system security, a kernel-mode ATD driver component should not assume that the commands that it receives necessarily come from legitimate user-mode ATD components. The kernel-mode component should perform complete verification of all command parameters and not rely on the user-mode component to do this.

A hardware vendor distributes the user-mode part of an ATD driver as a .dll file, and the kernel-mode part of the ATD driver as a .sys file.

# Call to Action and Resources

#### Call to Action:

For system manufacturers:

- Include audio transform devices and ATD drivers in your product plans

- Encourage audio device vendors to build audio transform devices that can be managed by ATD drivers

For audio device manufacturers:

- Include audio transform devices and ATD drivers in your product plans

- Contact Microsoft for information about designing audio transform devices that can be managed by ATD drivers

For driver developers:

- Include ATD drivers in your product plans

- Encourage audio device manufacturers and system manufacturers to build and use audio transform devices that can be managed by ATD drivers

For questions about ATD drivers, please send e-mail to atdfb@microsoft.com.

#### Resources

Universal Audio Architecture:

- For a general discussion of UAA drivers and hardware requirements, see:
The Universal Audio Architecture 
http://www.microsoft.com/hwdev/tech/audio

- For a discussion of USB Audio drivers and hardware requirements, see:
USB Audio Devices and Windows 
http://www.microsoft.com/hwdev/tech/audio

- For a discussion of IEEE 1394 audio drivers and audio hardware requirements, see:
IEEE 1394 AV/C Audio Devices and Windows 
http://www.microsoft.com/hwdev/tech/audio

DirectSound:

- For information about DirectSound, DirectX 8, and DMOs, see the Platform SDK documentation

- For information about audio driver support for DirectSound, see the Windows DDK documentation

Additional Resources and Tools:

Designed for Microsoft Windows XP Application Specification

http://www.microsoft.com/winlogo/software/windowsxp-sw.mspx

Microsoft Hardware and Driver Developer Information

http://www.microsoft.com/hwdev/

Microsoft Platform Software Development Kit (SDK)

http://msdn.microsoft.com/downloads/

Microsoft Windows Driver Development Kit (DDK)

http://www.microsoft.com/ddk/

Microsoft Windows Logo Program System and Device Requirements, version 2.1a

http://www.microsoft.com/winlogo/hardware/default.mspx

Windows XP Application Compatibility Toolkit

http://msdn.microsoft.com/compatibility/

Microsoft Windows XP Hardware Compatibility Test Kit, version 11.2

http://www.microsoft.com/hwdq/hwtest/