Getting Started with WDM Audio Drivers

WinHEC 2004 Version - April 13, 2004

Abstract

This paper provides information about Windows Driver Model (WDM) audio device drivers for the Microsoft® Windows® family of operating systems. It provides guidelines to hardware vendors for choosing how to supply driver support for their audio devices.

The paper primarily focuses on drivers that support PCI audio adapters by communicating with the system-supplied PortCls system driver through well-defined device driver interfaces. In addition, it briefly discusses driver issues for audio devices on external buses, such as the Universal Serial Bus (USB) and IEEE 1394. The general hardware requirements for Universal Audio Architecture (UAA) compliance are also discussed. A UAA-compliant USB Audio, IEEE 1394 AV/C, or High Definition (HD) Audio device can rely entirely on the operating system for driver support.

This information applies to the following operating systems:
	Microsoft Windows 98
	Microsoft Windows 2000
	Microsoft Windows Millennium Edition
	Microsoft Windows XP
	Microsoft Windows Server™ 2003
	Microsoft Windows codename “Longhorn”

Future versions of this preview information will be provided in the Windows Longhorn Driver Kit.

The current version of this paper is maintained on the Web at: 
http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

Contents

Introduction	3

Universal Audio Architecture	3

Custom Audio Drivers	4

Choosing a Wave Port Driver	5

WaveCyclic	6

WavePci	6

WaveRT	7

Supporting More Than One Wave Port Driver	7

Deciding Whether to Use the MIDI or DMus Port Driver	8

Specifying the Topology	8

Specifying Pin Data Ranges	11

References	12

Disclaimer

This is a preliminary document and may be changed substantially prior to final commercial release of the software described herein.

The information contained in this document represents the current view of Microsoft Corporation on the issues discussed as of the date of publication. Because Microsoft must respond to changing market conditions, it should not be interpreted to be a commitment on the part of Microsoft, and Microsoft cannot guarantee the accuracy of any information presented after the date of publication.

This White Paper is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS, IMPLIED OR STATUTORY, AS TO THE INFORMATION IN THIS DOCUMENT.

Complying with all applicable copyright laws is the responsibility of the user. Without limiting the rights under copyright, no part of this document may be reproduced, stored in or introduced into a retrieval system, or transmitted in any form or by any means (electronic, mechanical, photocopying, recording, or otherwise), or for any purpose, without the express written permission of Microsoft Corporation.

Microsoft may have patents, patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. Except as expressly provided in any written license agreement from Microsoft, the furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property.

Unless otherwise noted, the example companies, organizations, products, domain names, e-mail addresses, logos, people, places and events depicted herein are fictitious, and no association with any real company, organization, product, domain name, email address, logo, person, place or event is intended or should be inferred.

© 2004 Microsoft Corporation. All rights reserved.

Microsoft, Windows, Windows Server, DirectX, DirectMusic, and DirectSound are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

The names of actual companies and products mentioned herein may be the trademarks of their respective owners.

Introduction

This paper provides information about audio device drivers for the Microsoft® Windows® family of operating systems. It presents a step-by-step decision-making process that a hardware vendor can follow to decide how to provide driver support for an audio device on a computer running Windows.

In Windows 98, Windows Millennium Edition (Me), Windows 2000, Windows XP, Microsoft Windows Server™ 2003, and Windows codename “Longhorn,” audio drivers conform to the Windows Driver Model (WDM) and make use of the kernel streaming (KS) components, which operate in kernel mode and are part of the operating system.

Hardware vendors should make several design decisions before beginning development of a Windows-based audio hardware device.

The first decision is whether to design an audio device that requires a vendor-supplied custom driver. Windows contains operating-system support for PCI, USB, and IEEE 1394 devices that conform to the Universal Audio Architecture (UAA) guidelines, which were developed by Microsoft Corporation. The vendor does not need to provide a custom driver for an audio device that is UAA compatible.

However, if a vendor-supplied custom audio driver is necessary, the vendor must choose whether the driver should be designed to work in conjunction with the PortCls system driver (portcls.sys) or the AVStream class system driver (ks.sys). Both PortCls and AVStream are part of the Windows operating system. PortCls is the correct choice for most audio adapters.

When designing a custom adapter driver that uses PortCls, the vendor must decide how to make the devices on the audio adapter available to applications. Perhaps the most critical decision is selecting the appropriate port driver for the wave device of the audio adapter. PortCls provides three such drivers to choose from.

Two additional decisions involve how to present the adapter topology and pin data ranges to audio applications. The topology is a logical map of the data paths and control nodes in the adapter circuitry. The data ranges specify the data formats that the devices can support in their wave and MIDI streams. Both decisions affect how the devices on the audio adapter appear to applications.

In making all of the previously mentioned decisions, the hardware vendor must weigh the value of performance enhancements against the cost of implementing them. Another consideration is whether a particular solution can be made to work on a number of products in the Windows family. This paper provides an overview of these issues as well as references to more detailed documentation about specific topics.

For more information about WDM, KS, PortCls, and AVStream, see the Windows Device Driver Kit (DDK) documentation.

# Universal Audio Architecture

Audio devices that comply with the Universal Audio Architecture (UAA) guidelines, which were developed by Microsoft Corporation, can rely entirely on the operating system for driver support. The hardware vendor does not need to provide a custom driver for a UAA-compatible device. The UAA guidelines contain hardware requirements for audio devices that connect to the system bus (PCI) and to external buses (USB and IEEE 1394).

To be UAA-compatible, a PCI audio adapter must conform to the Intel High Definition (HD) Audio specification, which is the successor to the Intel AC'97 specification. Unlike AC'97, HD Audio standardizes the bus controller in addition to the codec devices and the serial interface link that connects the controller to the codecs. The UAA design guidelines for HD Audio devices contain additional requirements that are not part of the HD Audio specification. Microsoft® Windows® codename “Longhorn” provides complete driver support for UAA-compliant PCI audio adapters. In addition, UAA driver support for HD Audio devices can be obtained from Microsoft Corporation for Windows 2000, Windows XP, and Microsoft Windows Server™ 2003.

To be UAA-compatible, a USB Audio device must conform to both the USB Audio specification and the UAA design guidelines for USB Audio devices. USB Audio devices can make use of the USBAudio class system driver (usbaudio.sys), which is supplied as part of Windows 98, Windows Me, Windows 2000, Windows XP, Windows Server 2003, and Windows Longhorn. By definition, a USB Audio device that is compatible with the USBAudio class system driver in Windows Longhorn is UAA compliant.

To be UAA-compatible, an IEEE 1394 AV/C audio device must conform to both the relevant IEEE 1394 specifications and the UAA design guidelines for IEEE 1394 AV/C audio devices. IEEE 1394 audio devices can make use of the AVCAudio class system driver (avcaudio.sys), which is supplied as part of Windows XP, Windows Server 2003, and Windows Longhorn. By definition, an IEEE 1394 audio device that is compatible with the AVCAudio class system driver in Windows Longhorn is UAA compliant.

For more information about UAA hardware requirements for HD Audio, USB Audio, and IEEE 1394 AV/C audio devices, see the “References” section at the end of this paper.

# Custom Audio Drivers

Audio devices that are not UAA-compatible require vendor-supplied custom drivers. In addition, a UAA-compatible audio adapter can incorporate proprietary features that are not supported by the UAA class drivers; these features are accessible to applications only if the vendor provides a custom audio driver. Only the standard UAA features are accessible through the system-supplied UAA drivers. For information about UAA-supported features, see the “References” section at the end of this paper.

Two options are available to hardware vendors for writing custom audio drivers: developing a custom audio-adapter driver for use with the PortCls system driver (portcls.sys), or developing a custom minidriver for use with the AVStream class system driver (ks.sys).

Most custom drivers for audio adapters use PortCls, which is supplied as part of the operating system. PortCls contains a built-in audio-driver infrastructure that makes the task of writing a custom audio driver easier. PortCls implements several port drivers, each of which is specialized to manage the generic functions of a particular type of wave, MIDI, or mixer device. After selecting an appropriate set of port drivers to manage the audio functions on the audio adapter, the vendor develops a complementary set of miniport drivers that work in conjunction with the selected port drivers and control the hardware-dependent features of the audio devices.

The vendor can also support an audio device by developing a custom AVStream class minidriver. The minidriver works in conjunction with the AVStream class system driver, which is supplied as part of the operating system. Implementing an AVStream driver is more difficult than using PortCls, but doing so might still be appropriate for devices that integrate audio and video. An AVStream driver might also be necessary for an existing USB or IEEE 1394 audio device that fails to comply with the requirements of the system-supplied USBAudio or AVCAudio class system driver.

For nearly all PCI audio adapters that require vendor-supplied custom drivers, vendors should choose PortCls.

The PortCls system driver (portcls.sys) is available on Microsoft® Windows® 98, Windows Me, Windows 2000, Windows XP, Microsoft Windows Server™ 2003, and Windows codename “Longhorn.” The device driver interfaces (DDIs) in PortCls are essentially the same in all these Windows products. By relying on PortCls, a vendor can therefore write a single adapter driver that works in all these Windows products.

The AVStream class system driver (ks.sys) is available in Windows XP, Windows Server 2003, and Windows Longhorn. AVStream is also available in Windows 98, Windows Me, and Windows 2000, but only if Microsoft DirectX® version 8.0 or later is installed. However, AVStream lacks most of the audio-specific support functions that exist in PortCls.

In Windows 2000, Windows Me, and Windows 98, the Stream class system driver (stream.sys) provides general streaming support for audio and video devices. In Windows XP and later, Stream has been superseded by AVStream. Microsoft currently supports Stream on an as-is basis for the benefit of existing Stream class minidrivers.

For more information about PortCls, AVStream, and Stream, see the Windows DDK documentation.

# Choosing a Wave Port Driver

After deciding to write a custom driver that uses PortCls, the hardware vendor’s next step is to choose one of the system-supplied wave port drivers in PortCls. This is the driver that performs all the generic control functions for the wave device in your audio adapter. By definition, a wave device renders and captures digital audio streams that have data formats specified by the WAVEFORMATEX and WAVEFORMATEXTENSIBLE structures. For information about these structures, see the Microsoft® Windows® DDK documentation.

The hardware vendor must supply the custom miniport driver that works in conjunction with the system-supplied port driver.

In Windows 98, Windows Me, Windows 2000, Windows XP, and Microsoft Windows Server™ 2003, PortCls provides two wave port drivers: WavePci and WaveCyclic. In Windows codename “Longhorn,” PortCls provides a third wave port driver, named WaveRT, in addition to WavePci and WaveCyclic.

After selecting a WaveXxx (that is, WavePci, WaveCyclic, or WaveRT) port driver, the hardware vendor must write a complementary WaveXxx miniport driver to control the device-specific functions on behalf of the port driver. Each port driver both exposes a different software interface to the miniport driver and requires the miniport driver to implement and expose a different interface to the port driver.

WavePci is the preferred port driver in Windows 98, Windows Me, Windows 2000, Windows XP, and Windows Server 2003, because WavePci offers better audio performance than WaveCyclic does. However, on these Windows products, most existing audio devices use WaveCyclic instead of WavePci. The reason is that WaveCyclic has hardware requirements that are less demanding, and it has a simpler miniport-driver interface.

In Windows Longhorn, WaveRT is the preferred wave port driver. Although Windows Longhorn will continue to support existing WavePci and WaveCyclic miniport drivers, new miniport drivers can achieve better performance by using the WaveRT port driver.

The following sections briefly describe the three WaveXxx port drivers.

## WaveCyclic

The WaveCyclic port driver provides support for a wave audio device by using a simple cyclic audio buffer. WaveCyclic is appropriate for audio devices with limited direct memory access (DMA) capabilities, including:

- Devices that require the cyclic buffer to occupy a single, contiguous block of physical memory

- Devices that cannot access addresses everywhere in physical memory

The drawback to the WaveCyclic port driver is that it must copy of each byte of data in the audio stream. WaveCyclic must copy data between its cyclic buffer and the application's audio buffer. WaveCyclic can allocate the cyclic buffer as a contiguous block of physical memory so that the audio hardware can directly access the buffer. However, the application allocates and accesses its own audio buffer, which is contiguous in virtual memory but not in physical memory.

WaveCyclic can also support older audio devices that lack DMA capabilities and must rely on the system DMA controller for streaming audio data to and from the hardware.

For more information about the WaveCyclic port driver, see the Windows DDK documentation.

## WavePci

By avoiding the need to copy audio data, the WavePci port driver typically consumes less processor time than the WaveCyclic port driver. However, WavePci is more demanding in terms of audio hardware capabilities. To directly access the noncontiguous pages of physical memory in the application's audio buffer, the audio device must be able to perform the following:

- Scatter-gather DMA transfers

- DMA transfers to and from any location in physical memory

The audio device accesses the application's buffer as a series of mappings, each of which consists of a contiguous chunk of physical memory. To acquire a mapping, the miniport driver calls the port driver's GetMapping method; it calls the port driver's ReleaseMapping method when it finishes processing the mapping. During the time that the miniport driver holds each mapping, the physical memory page containing the mapping remains locked.

A WavePci miniport driver is more complicated than its WaveCyclic counterpart. When implementing a WavePci miniport driver, a hardware vendor must pay careful attention to synchronization issues. During a call to the port driver to acquire or release a mapping, the miniport driver cannot hold any spin locks. This requirement effectively prevents the miniport driver from maintaining exclusive access to any shared data structure during the call. In addition, a WavePci miniport driver must be prepared to revoke at any time any mappings that it holds. Mappings are revoked when a client cancels an I/O request packet that contains audio data.

For more information about the WavePci port driver, see the Windows DDK documentation.

## WaveRT

The WaveRT port driver has performance advantages over both the WavePci and the WaveCyclic port drivers. WaveRT avoids any need for intervention by the miniport driver in the flow of audio data between the application and the audio hardware. This feature is particularly important for clients running Windows Longhorn that use real-time threads to reduce audio stream latencies. In contrast, while streaming audio data, a WaveCyclic miniport driver must copy data, and a WavePci miniport driver must continually acquire and release mappings that contain audio data.

A WaveRT miniport driver is similar in complexity to a WaveCyclic miniport driver. Unlike WavePci and WaveCyclic miniport drivers, a WaveRT miniport driver is responsible for allocating the physical memory pages that comprise the audio buffer used by the application. By controlling buffer allocation, the miniport driver can compensate for any limitations in its DMA hardware. If the device cannot perform scatter-gather DMA transfers, the driver can allocate a contiguous block of physical memory for the buffer. If the device cannot access all of physical memory, the driver can allocate the buffer from the physical memory that it can access.

Thus, existing audio hardware with these kinds of limitations can use the WaveRT port driver. However, new WaveRT-compatible audio hardware should be designed to support scatter-gather DMA transfers, addressing that is at least 32-bit, and a wall clock register and DMA position registers that can be mapped to user-mode virtual memory. For more information, see the white paper titled A Wave Port Driver for Real-Time Audio Streaming at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

The WaveRT port driver is available only in Windows Longhorn.

## Supporting More Than One Wave Port Driver

Instead of developing a single wave miniport driver to run on all Windows products, a hardware vendor might choose to write two or three miniport drivers and then pick the driver that is the most appropriate to run on the target system. During device installation and startup, the vendor has considerable flexibility in deciding which driver to use.

For example, a WaveRT miniport driver is typically the best choice for a computer running Windows Longhorn. By using decorated section names that designate specific platforms, the hardware vendor can write an INF file that installs the WaveRT miniport driver only if the computer is running Windows Longhorn. If the computer is running another Windows product, the INF file will install a WavePci or WaveCyclic miniport driver instead.

For another example, assume that an audio device generates a 32-bit DMA address, performs scatter-gather DMA transfers, and resides on an audio adapter in a 64-bit system. The hardware vendor might provide both WavePci and WaveCyclic miniport drivers for the device. The adapter driver can decide which miniport driver to use at the time that the operating system tells it to start the device (in other words, when the operating system sends an IRP_MN_START_DEVICE request). If the adapter driver determines that it can access all the physical memory in the system, it will create an instance of the WavePci miniport driver to control the device. If the device cannot access some portion of physical memory, the adapter driver will create an instance of the WaveCyclic miniport driver instead.

# Deciding Whether to Use the MIDI or DMus Port Driver

PortCls provides a MIDI port driver and a DMus port driver. The MIDI port driver controls a simple MIDI device, such as an FM synthesizer or MPU-401 port. The DMus port driver is an enhanced version of the MIDI port driver that supports advanced MIDI features, such as downloadable sounds (DLS), precision sequencing, and channel groups that are used by Microsoft® DirectMusic® applications.

Hardware support for MIDI and DirectMusic generally falls into two categories. The first category consists of MPU-401 ports for sending MIDI streams to external MIDI devices and for capturing MIDI streams from external devices. Most systems have audio adapters, but few are connected to external MIDI devices. An inexpensive audio adapter might omit the MPU-401 ports to reduce cost.

The second category consists of synthesizers for playing MIDI streams. For older systems with limited processing power, hardware synthesizers are useful for faithfully rendering high-quality MIDI streams that contain many instruments and require precision timing. With today's faster processors and improved operating systems, software synthesizers are able to satisfy all but the most demanding MIDI-rendering requirements.

The MIDI and DMus miniport drivers make MIDI hardware capabilities available to applications that use DirectMusic and the Windows® multimedia midiInXxx and midiOutXxx functions in Microsoft Windows 98, Windows Me, Windows 2000, Windows XP, and Microsoft Windows Server™ 2003.

In Windows codename “Longhorn,” DirectMusic and the midiInXxx and midiOutXxx functions will increasingly rely on software synthesis as a means of enforcing the operating system's global policy on MIDI streams. Audio applications such as acoustic echo cancellation (AEC) depend on strict adherence to global policy. The degree to which applications running in Windows Longhorn can make use of MIDI hardware is still being determined.

For more information about MIDI and DMus port drivers and miniport drivers, see the Windows DDK documentation. For more information about DirectMusic and Windows multimedia, see the Windows Platform software development kit (SDK) documentation.

# Specifying the Topology

After deciding which miniport drivers to write for your wave and MIDI devices, the next step is to represent the kernel streaming (KS) topology of these devices. The KS topology consists of a set of data structures describing the data paths that audio or MIDI streams follow as they flow through each device. Through this topology, the driver exposes the control nodes that lie along each path. Typically, an application uses the Microsoft® Windows® multimedia mixerXxx functions to explore the topology by enumerating the sequence of nodes along each path. For example, after discovering a volume-level control node, an application can set the volume level on that node. For more information about Windows multimedia, see the Windows Platform software development kit (SDK) documentation.

PortCls provides six port drivers: WavePci, WaveCyclic, WaveRT, MIDI, DMus, and Topology. The Topology port driver controls the portion of the audio adapter circuitry that mixes together the rendering streams from the wave and MIDI devices. It also controls the selection of capture streams from input jacks. In spite of its somewhat misleading name, the Topology port driver does not embody all of an audio adapter's topology, although it does contain a large portion of it. The other port drivers contribute the remaining portions of the adapter's topology.

Each port driver is paired with a corresponding miniport driver to form a KS filter that represents a particular device (wave, MIDI, or mixer) on the audio adapter, as shown in the following table.

Filter Types

| Type | Description |
| --- | --- |
| WaveXxx filter | Represents a wave device that converts a wave output stream to an analog audio signal or vice versa |
| MIDI or DMus filter | Represents a MIDI device that plays or captures a MIDI stream |
| Topology filter | Represents the adapter's mixer circuitry |

Typically, the pins on the Topology filter are tied to the pins on the wave, MIDI, and DMus filters through hardwired connections in the adapter circuitry. These filters and their interconnections together form a KS filter graph that embodies the adapter's topology.

Figure 1 shows the topology of an example audio adapter.

Figure 1. Topology Example

In Figure 1, the topology at the top level consists of the connections among the MIDI, WaveXxx, and Topology filters. In addition, each filter has its own internal topology, which consists of the data paths through the filter and the nodes that lie along each path. The nodes are labeled as shown in the following table.

Topology Nodes

| Label | Description | KS node type GUID |
| --- | --- | --- |
| Synth | Synthesizer node | KSNODETYPE_SYNTHESIZER |
| DAC | Digital-to-audio converter node | KSNODETYPE_DAC |
| ADC | Analog-to-digital converter node | KSNODETYPE_ADC |
| Volume | Volume-level control node | KSNODETYPE_VOLUME |
| Mute | Mute control node | KSNODETYPE_MUTE |
| Sum | Summation node | KSNODETYPE_SUM |
| MUX | Multiplexer node | KSNODETYPE_MUX |

The pins on the left side of the audio adapter represent the logical points (not the physical connections) through which data streams enter the adapter from the system bus or enter the system bus from the adapter. These pins are logically connected to source and sink pins on other filters (not shown) that are external to the adapter. Typically, these filters are software modules that, together with the adapter topology, form a larger filter graph whose topology can be explored by applications using the mixerXxx functions. For example, the PCM wave output pin in Figure 1 is logically connected to the global audio engine (in Windows codename “Longhorn”) or to the kernel mixer (in Microsoft Windows Server™ 2003, Windows XP, Windows 2000, Windows Me, and Windows 98). These logical connections are maintained by DMA transfers over the system bus.

In contrast, the pins on the left edge of the Topology filter are physically connected to pins on the MIDI and WaveXxx filters. These connections are hardwired and cannot be changed by software.

The pins on the right side of the adapter represent audio jacks on the system chassis. These pins are referred to as bridge pins because they bridge the boundary between the KS filter graph and the external world.

Filters, pins, and nodes typically have properties that are accessible to clients (kernel-mode components or user-mode applications) of the audio driver. A client can send a KS property request to a filter, pin, or node either to query for the current value of a property or to change the property value. For example, a volume-level control node has a KSPROPERTY_AUDIO_VOLUMELEVEL property, which a client can change through a KS property request. A MUX node is an example of a node type that typically has no properties.

For simplicity, the WaveXxx filter in Figure 1 provides only a single pin for accepting a PCM wave output stream from the system bus. In contrast, some wave devices provide multiple pins for PCM wave output and contain hardware for internally mixing the streams that enter the pins. These devices provide hardware acceleration for applications that use Microsoft DirectSound® by accepting PCM streams that play from the applications’ sound buffers. For DirectSound to use these pins, they must provide additional nodes for two-dimensional (2-D) and three-dimensional (3-D) processing, as described in the Windows DDK documentation.

This type of hardware acceleration is supported in Windows Server 2003, Windows XP, Windows 2000, Windows Me, and Windows 98, but it is not supported in Windows Longhorn. Windows Longhorn uses audio transform device (ATD) drivers to accelerate applications that use either DirectSound or the other audio APIs, but it makes no use of the hardware acceleration pins on older wave devices. No other Windows products support ATD drivers. For more information about ATD drivers, see the “References” section at the end of this paper.

In Figure 1, the physical connections between the MIDI, WaveXxx, and Topology filters all transport analog audio signals. However, a different Topology device might achieve a similar effect by accepting digital output streams from the MIDI and wave devices, digitally mixing them, and converting the digital mix to an analog output signal.

The pin at the bottom-left corner of Figure 1 accepts a non-PCM output stream in an S/PDIF pass-through format, such as AC-3−over−S/PDIF or WMA Pro-over-S/PDIF. Using one of these formats, the device simply transmits the compressed data over the S/PDIF link without decoding the data. For this reason, the data path to the S/PDIF output pin on the right edge of Figure 1 contains no volume or mute nodes. For more information about non-PCM audio formats and S/PDIF pass-through transmission, see the Windows DDK documentation. Additional information is available in the white paper titled Audio Driver Support for the WMA Pro-over-S/PDIF Format at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

The miniport driver presents its topology to the port driver in the form of a PCFILTER_DESCRIPTOR structure. This structure describes all the filter's pins and nodes, and it specifies how the pins and nodes connect to each other. For detailed information about the PCFILTER_DESCRIPTOR structure, see the Windows DDK documentation.

# Specifying Pin Data Ranges

After defining a topology that represents the data paths and control nodes in your devices, the next step is to define the data ranges for each configurable pin. A configurable pin can be created, configured, and connected to a wave or MIDI stream under software control. In contrast, a physical connection or bridge pin exists implicitly and can neither be created nor configured under software control.

Before connecting a configurable pin to serve as a sink or source for a wave or MIDI stream, the pin must be configured to handle the data format for the stream. Typically, the pin can be configured to accept one of several stream formats. For example, a PCM wave-output pin might accept the following ranges of PCM stream parameters:

- Sample rates of 11.025 kHz, 22.05 kHz, 44.1 kHz, and 48 kHz

- Sample sizes of 8, 16, 24, and 32 bits

- Any number of channels from 1 through 8

For each type of configurable pin, a miniport driver describes the various stream data formats that the pin can handle. These parameter ranges can be specified as an array of data-range descriptors, as shown in the following code example.

Static KSDATARANGE_AUDIO PinDataRangesPcm[] =

{

{

{

sizeof(KSDATARANGE_AUDIO),

0,

0,

0,

STATICGUIDOF(KSDATAFORMAT_TYPE_AUDIO),

STATICGUIDOF(KSDATAFORMAT_SUBTYPE_PCM),

STATICGUIDOF(KSDATAFORMAT_SPECIFIER_WAVEFORMATEX)

},

8,       // Maximum number of channels

8,       // Minimum number of bits per sample

32,      // Maximum number of bits per channel

11025,   // Minimum rate

48000    // Maximum rate

}

};

Note that the PinDataRangesPcm array in the preceding example contains a single data-range descriptor of type KSDATARANGE_AUDIO. More generally, a data-range array can contain an arbitrary number of descriptors. For example, a non-PCM wave-output pin might support both AC-3−over−S/PDIF and WMA Pro-over-S/PDIF formats. Each of these two formats is specified by a separate data-range descriptor. Thus, the pin's data-range array would contain at least two KSDATARANGE_AUDIO structures.

A configurable pin that supports the music stream format from an application that uses Microsoft® DirectMusic® or the Windows® multimedia midiInXxx and midiOutXxx functions uses a data-range descriptor of type KSDATARANGE_MUSIC.

For information about the KSDATARANGE_AUDIO and KSDATARANGE_MUSIC structures, see the Microsoft Windows DDK documentation.

The port driver obtains the data-range information from the miniport driver and uses this information, wherever possible, to handle requests for information about the data formats that each pin can support. In the case of a pin with a simple PCM data range, the port driver is able to handle the intersection requests for that pin. In an intersection request, a client provides a set of data ranges that represent possible data formats for a stream. If possible, the port driver's intersection handler picks a particular data format from the data ranges in the request that also falls within its pin's data ranges. This format represents an intersection of the two sets of data ranges. Hence, both the client and the pin can process a stream with this format. For more complex data ranges, the miniport driver can provide its own intersection handler, which the port driver then uses instead of its own, default handler. The intersection handler of the miniport driver can allow for any format requirements that might be difficult to express to the port driver as an array of data ranges. For more information about intersection handlers, see the Windows DDK documentation. Additional information is available in the white paper titled Multiple Channel Audio Data and WAVE Files at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

# References

Call to Action

- For system manufacturers: When selecting a USB or IEEE 1394 audio device, be sure that the device is compatible with the system-supplied USBAudio or AVCAudio class system driver. When selecting a PCI audio adapter, consider using a UAA-compatible device, which does not require a vendor-supplied custom driver. If the adapter has features that require a custom driver, be sure that the driver efficiently works with the PortCls system driver.

- For device manufacturers: When designing a USB or IEEE 1394 audio device, be sure that the device complies with the UAA requirements for the system-supplied USBAudio or AVCAudio class system driver. When designing a PCI audio device, be sure that the device is UAA-compatible and that any additional, non-UAA features can be efficiently supported by a vendor-supplied custom driver that works with the PortCls system driver. In particular, avoid implementing proprietary hardware features in a way that prevents the system-supplied UAA class driver from supporting your device.

- For driver developers: When writing a PortCls-compatible driver for a PCI audio adapter, be sure that you pick the best set of port and miniport drivers to control the devices on the adapter. Also, pick a suitable topology to represent the adapter's internal data paths and, for each pin, choose an appropriate set of data ranges that specify the data formats supported by the adapter.

- If you have questions about the Universal Audio Architecture initiative and how it pertains to various audio device technologies, send e-mail to uaa@microsoft.com.

Resources

PCI Audio Adapters

For information about the PortCls system driver and the AVStream class system driver, see the Microsoft® Windows® DDK documentation.

For information about the WaveRT port driver, see the white paper titled A Wave Port Driver for Real-Time Audio Streaming at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

Universal Audio Architecture

For information about the Universal Audio Architecture (UAA) initiative, which was developed by Microsoft Corporation, for USB Audio, IEEE 1394 AV/C, and High Definition (HD) Audio devices, see the white paper titled Universal Audio Architecture: Compatibility Design Guidelines at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

For information about the High Definition Audio specification, developed by Intel, send e-mail to nextgenaudio@intel.com.

USB Audio

For information about support that Windows provides for USB Audio devices, see the white paper titled USB Audio Devices and Windows at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

For all public USB specifications, see the USB Implementers Forum Web site at http://www.usb.org. The following specifications apply to devices that use the USBAudio class system driver:

- Universal Serial Bus Device Class Definition for Audio Devices, Release 1.0

- Universal Serial Bus Device Class Definition for Audio Data Formats, Release 1.0

- Universal Serial Bus Device Class Definition for Terminal Types, Release 1.0

- Universal Serial Bus Device Class Definition for MIDI Devices, Release 1.0

IEEE 1394 Audio

For information about support that Windows provides for IEEE 1394 audio devices, see the white paper titled IEEE 1394 AV/C Audio Devices and Windows at http://www.microsoft.com/whdc/hwdev/tech/stream/1394-AVC.mspx.

For IEEE 1394 audio specifications, see http://www.1394ta.org. The specifications at this site are available to members of the 1394 Trade Association. The following specifications apply to devices that use the AVCAudio class system driver:

- AV/C Audio Subunit Specification 1.0

- IEEE 61883-1 and IEEE 61883-6 Protocol Specifications (IEC 60958)

- AV/C Digital Interface Command Set General Specification 3.0

- AV/C Stream Format Information Specification 1.0

- AV/C Connection and Compatibility Management Specification 1.0

ATD Drivers

For information about audio transform devices (ATDs) and ATD drivers, see the white paper titled Audio Transform Device Drivers at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

Multiple-Channel Audio Formats

For information about multiple-channel audio formats, see the white paper titled Multiple Channel Audio Data and WAVE Files at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

Non-PCM Audio Formats

For information about non-PCM audio formats, see the white paper titled Non-PCM Wave Formats and WDM Audio Drivers at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

WMA Pro-over-S/PDIF Format

For information about transmitting a WMA Pro audio stream through an S/PDIF connection, see the white paper titled Audio Driver Support for the WMA Pro-over-S/PDIF Format at http://www.microsoft.com/whdc/hwdev/tech/audio/default.mspx.

Microsoft Hardware and Driver Central

(includes Windows Driver Development Kits [DDKs], Windows Hardware Compatibility Test [HCT] Kits, and Windows Logo Program requirements)
http://www.microsoft.com/whdc/default.mspx